Tomasulo Algorithm Project

ECE6100

Bihai Zhang

GT ID: 903547409

- Summary of Tomasulo Algorithm

Tomasulo algorithm is a hardware algorithm for dynamic scheduling of instructions. As a result, the hardware can achieve out-of-order execution of instructions. So, the execution units are more efficiently utilized instead of wasting time in idle status. The core innovation of this algorithm is register renaming in hardware. Robert Tomasulo invented reservation stations to support register renaming in hardware. And a common data bus (CDB) was introduced to let each reservation station receive operands.

Figure 1 Tomasulo Algorithm

As is shown in Figure 1, instructions are loaded to corresponding reservations stations in a FIFO order. The reservation station store the operation to perform, the value of operands/where the operands will be generated. Firstly, a RS issue an instruction. Then, when all operands are available, execution of this instruction will begin in the cycle following the cycle when all its operands are known, assuming that computing resources are available. Finally, when the execution ends, if the CDB is available, the computing unit will broadcast the result to all RS via the CDB. Each RS will check if the value is needed.

The program uses a main loop to simulate the cycles of the processing unit. In each cycle, the program will make decisions according to the rules set by the Tomasulo algorithm.

- Resource Code

The resource code is appended in the Appendix.

- Example Runs

In the following subsections, RS Add1, Add2, Add3, Mult1, Mult2, Load1, Load2, Load3, Stor1, Stor2, Stor3 are numbered as 1,2,3,4,5,6,7,8,9,10,11 in the register result status. The default value stored in R0, R2, R4, R6, R8 and R10 are 1, 2, 3 ,4, 5 and 6. Cycles required for execution are assumed to be the same as the long example.

  - WAR

Instructions:

ADDD R0 R4 R2

ADDD R4 R0 R8

The second instruction must write to R4 after the first instruction read the value in R4. So, it is WAR.

Cycle 1

Cycle 2

Cycle 3

Cycle 4

Cycle 5

Cycle 6

Cycle 7

  - RAW

Instructions:

ADDD R2 R0 R4

ADDD R8 R0 R2

The second instruction must read the value in R2 after the first instruction writes to R2. So, it is RAW.

Cycle 1

Cycle 2

Cycle 3

Cycle 4

Cycle 5

Cycle 6

Cycle 7

  - WAW

Instructions:

ADDD R4 R0 R6

ADDD R2 R0 R4

ADDD R4 R0 R8

There is WAW hazard via R4 between the first instruction and the last instruction.

Cycle 1

Cycle 2

Cycle 3

Cycle 4

Cycle 5

Cycle 6

Cycle 7

  - The Long Example

Instructions:

LD R6 10

LD R2 12

MULTD R0 R2 R4

SUBD R8 R6 R2

DIVD R10 R0 R6

ADDD R6 R8 R2

For simplicity, it is assumed that the LD operation load an immediate directly to the register.

Cycle 1

Cycle 2

Cycle 3

Cycle 4

Cycle 5

Cycle 6

Cycle 7

Cycle 8

Cycle 9

Cycle 10

Cycle 11

Cycle 12

Cycle 13

Cycle 14

Cycle 15

Cycle 16

Cycle 17

Cycle 18

From cycle 18, only the DIVD instruction is in operation. For simplicity, the intermediate steps will be omitted.

Cycle 57

In this cycle, the execution of DIVD instruction is completed.

Cycle 58

  - Example with Stalls

Instructions:

MULTD R0 R2 R4

ADDD R6 R8 R10

LD R8 20

The LD instruction wants to load a value to R8. Although the load buffer is available, it cannot be executed until ADDD has read the value in R8. So, there is stall for the LD instruction.

Cycle 1

Cycle 2

Cycle 3

Cycle 4

Cycle 5

Cycle 6

Cycle 7

Cycle 8

From cycle 8, there is only the MULTD instruction in operation. For simplicity, let’s jump to cycle 11.

Cycle 11

Cycle 12

Appendix

- //

- //  main.cpp

- //  this program implements the tomasulo algorithm

- //  instruction types supported: LD, SD, MULTD, DIVD, ADDD, SUBD

- //  <store register>: [R0, Rn] where n is the number of registers (configured in assumptions)

- //  Note: the program takes advantage of strncmp with 2 characters to find the correct register,

- //  so if the number of registers is too large, the program may not execute correctly

- #include <iostream>

- #include <iomanip>

- #include <cmath>

- #include <fstream>

- #include <stdio.h>

- #include <string.h>

- #include <stdlib.h>

- #define MAXCHAR 1000

- using namespace std;

- // defining the data type for all instructions

- typedef struct instruction

- {

- char type[100];

- char dest_reg[100];

- char reg_j[100];

- char reg_k[100];

- double num;

- double rs=0;

- double issue=-1;

- int start=0;

- double completion=-1;

- double written=-1;

- int result;

- int load=0;

- bool completing=false;

- bool writing=false;

- } instruction;

- // defining the data type for memory

- typedef struct memory

- {

- char name[2];

- double data;

- int tag=0;

- bool busy=false;

- } memory;

- // defining the data type for all RS

- typedef struct reservation_station

- {

- char name[100]="NONE";

- double num;

- double data_j=NULL;

- double data_k=NULL;

- double result;

- int tag_j=0;

- int tag_k=0;

- int instr=-1;

- int cycle_count=0;

- int cycles_required=-999;

- bool executing=false;

- bool busy=false;

- } reservation_station;

- typedef struct load_store_rs

- {

- char name[100]="NONE";

- double num;

- double address=NULL;

- int instr=-1;

- int tag=0;

- int cycle_count=0;

- int cycles_required=-999;

- bool executing=false;

- bool busy=false;

- } load_store_rs;

- void header(int n);

- template<typename T> void printElement(T t, const int& width);

- template<typename T> void printInstructionStatus(T t, const int& width);

- template<typename T> void printLoadStatus(T t, const int& width);

- template<typename T> void printStoreStatus(T t, const int& width);

- template<typename T> void printStationStatus(T t, const int& width);

- template<typename T> void printRegisterStatus(T t, const int& width);

- int main() {

- // ==================== ASSUMPTIONS ====================

- // all assumptions are conssitent with the project description

- int addCycles = 2;

- int subCycles = 2;

- int multCycles = 10;

- int diviCycles = 40;

- int loadCycles = 3;

- int storeCycles = 3;

- const int addReservationStations = 3;

- const int mulReservationStations = 2;

- const int loadReservationStations = 3;

- const int storeReservationStations = 3;

- const int maxInstructions = 16;

- const int numRegisters = 6;

- // initializing the values in registers, add or delete entries depending on the number of registers

- float dataRegisters[numRegisters] = {1, 2, 3, 4, 5, 6};

- //char* filename = "raw.txt";

- //char* filename = "waw.txt";

- char* filename = "war.txt";

- //char* filename = "sdld.txt";

- //char* filename = "stall.txt";

- //char* filename = "long.txt";

- // ==================== STRUCTURE INITIALIZATION ====================

- instruction instruction_list[maxInstructions];

- memory  data_registers[numRegisters];

- char rs_num[3];

- for (int i=0; i < numRegisters; i++){

- strcpy(data_registers[i].name, "R");

- sprintf(rs_num, "%i", i*2);

- strcat(data_registers[i].name, rs_num);

- data_registers[i].data = dataRegisters[i];

- }

- reservation_station add_rs[addReservationStations];

- reservation_station mul_rs[mulReservationStations];

- int j;

- for (j=0; j < addReservationStations; j++) {

- add_rs[j].num = j + 1;

- }

- int k;

- for (k=0; k < mulReservationStations; k++) {

- // each reservation station needs a unique number

- mul_rs[k].num = j + k + 1;

- }

- load_store_rs load_rs[loadReservationStations];

- load_store_rs store_rs[storeReservationStations];

- for (int i=0; i < loadReservationStations; i++) {

- load_rs[i].num = addReservationStations + mulReservationStations + i + 1;

- }

- for (int i=0; i < storeReservationStations; i++) {

- store_rs[i].num = addReservationStations + mulReservationStations + loadReservationStations + i + 1;

- }

- // 1,2,3 add rs;4,5 mul rs;6,7,8 load rs;9,10,11 store rs

- // ==================== VARIABLE INITIALIZATION ====================

- int lineCount = 0;

- int completedInstr = 0;

- int issuedInstr = 0;

- int writtenInstr = 0;

- int clockCycles = 0;

- int completed_rs = -1;

- double cdb_data = 0;

- bool issueSuccessful = false;

- bool cdb_busy = false;

- // ==================== READ IN INSTRUCTIONS ====================

- FILE *fp;

- char mystring[MAXCHAR];

- fp = fopen(filename, "r");

- if (fp == NULL){

- printf("Could not open file %s",filename);

- return 1;

- }

- while (fgets(mystring, MAXCHAR, fp) != NULL){

- const char t[2] = "\t";

- const char s[2] = " ";

- char *token;

- //cpoying instruction type

- token = strtok(mystring, t);

- instruction_list[lineCount].num = atof(token);

- strcpy(instruction_list[lineCount].type, strtok(token, s));

- // copying destination register

- token = strtok(NULL, s);

- strcpy(instruction_list[lineCount].dest_reg, token);

- // copying operand j

- token = strtok(NULL, s);

- if (strcmp(instruction_list[lineCount].type, "LD") == 0) {

- instruction_list[lineCount].load = atoi(token);

- }

- else {

- strcpy(instruction_list[lineCount].reg_j, token);

- }

- /* read in last part of instruction, if not load or store */

- if (strcmp(instruction_list[lineCount].type, "LD") == 0 or strcmp(instruction_list[lineCount].type, "SD") == 0)

- {

- strcpy(instruction_list[lineCount].reg_k, " ");

- }

- else

- {

- token = strtok(NULL, s);

- strcpy(instruction_list[lineCount].reg_k, token);

- }

- lineCount += 1;

- }

- fclose(fp);

- // ==================== MAIN SIMULATION LOOP ====================

- while (writtenInstr < lineCount)

- {

- // ================== WRITING INSTRUCTIONS ==================

- if (completed_rs != -1) {

- // broadcasting data

- for (int i=0; i < addReservationStations; i++) {

- if (add_rs[i].tag_j == completed_rs) {

- add_rs[i].data_j = cdb_data;

- add_rs[i].tag_j = 0;

- }

- if (add_rs[i].tag_k == completed_rs) {

- add_rs[i].data_k = cdb_data;

- add_rs[i].tag_k = 0;

- }

- // clear reservation station

- if (add_rs[i].num == completed_rs) {

- add_rs[i].busy = false;

- add_rs[i].cycle_count = 0;

- add_rs[i].cycles_required = -999;

- add_rs[i].executing = false;

- }

- }

- for (int i=0; i < mulReservationStations; i++) {

- if (mul_rs[i].tag_j == completed_rs) {

- mul_rs[i].data_j = cdb_data;

- mul_rs[i].tag_j = 0;

- }

- if (mul_rs[i].tag_k == completed_rs) {

- mul_rs[i].data_k = cdb_data;

- mul_rs[i].tag_k = 0;

- }

- // clear reservation station

- if (mul_rs[i].num == completed_rs) {

- mul_rs[i].busy = false;

- mul_rs[i].cycle_count = 0;

- mul_rs[i].cycles_required = -999;

- mul_rs[i].executing = false;

- }

- }

- for (int i=0; i < loadReservationStations; i++) {

- if (load_rs[i].tag == completed_rs) {

- load_rs[i].address = cdb_data;

- load_rs[i].tag = 0;

- //load_rs[i].executing = true;

- }

- // clear reservation station

- if (load_rs[i].num == completed_rs) {

- load_rs[i].busy = false;

- load_rs[i].cycle_count = 0;

- load_rs[i].cycles_required = -999;

- load_rs[i].executing = false;

- }

- }

- for (int i=0; i < storeReservationStations; i++) {

- if (store_rs[i].tag == completed_rs) {

- store_rs[i].address = cdb_data;

- store_rs[i].tag = 0;

- //store_rs[i].executing = true;

- }

- // clear reservation station

- if (store_rs[i].num == completed_rs) {

- store_rs[i].busy = false;

- store_rs[i].cycle_count = 0;

- store_rs[i].cycles_required = -999;

- store_rs[i].executing = false;

- }

- }

- // write to memory

- for (int i=0; i < numRegisters; i++) {

- if (data_registers[i].tag == completed_rs) {

- data_registers[i].data = cdb_data;

- data_registers[i].busy = false;

- data_registers[i].tag = 0;

- }

- }

- // just for bookkeeping - instruction written cycle number

- for (int j=0; j < issuedInstr; j++) {

- if (instruction_list[j].rs == completed_rs) {

- if (instruction_list[j].written == -1) {

- instruction_list[j].written = clockCycles+1;

- }

- }

- }

- writtenInstr += 1;

- // reset

- completed_rs = -1;

- cdb_data = 0;

- }

- // ================== ISSUING INSTRUCTIONS ==================

- if (issuedInstr < lineCount) {

- // issue instruction 0...then 1...then n..etc. (& increment instruction cycle if successful)

- if (strcmp(instruction_list[issuedInstr].type, "ADDD") == 0 || strcmp(instruction_list[issuedInstr].type, "SUBD") == 0) {

- // if addition or subtraction, adding to reservation station

- for (int l=0; l < addReservationStations; l++) {

- if (issueSuccessful == false) {

- if (add_rs[l].busy == false) {

- for (int i=0; i < numRegisters; i++) {

- // j operand

- if (strcmp(instruction_list[issuedInstr].reg_j, data_registers[i].name) == 0) {

- add_rs[l].data_j = data_registers[i].data;

- if (data_registers[i].tag == 0) {

- }

- else {

- add_rs[l].tag_j = data_registers[i].tag;

- }

- }

- // k operand

- if (strncmp(instruction_list[issuedInstr].reg_k, data_registers[i].name, 2) == 0) {

- if (data_registers[i].tag == 0) {

- add_rs[l].data_k = data_registers[i].data;

- }

- else {

- add_rs[l].tag_k = data_registers[i].tag;

- }

- }

- // destination register

- if (strcmp(instruction_list[issuedInstr].dest_reg, data_registers[i].name) == 0) {

- data_registers[i].tag = add_rs[l].num;

- }

- }

- add_rs[l].busy = true;

- instruction_list[issuedInstr].rs = add_rs[l].num;

- add_rs[l].cycle_count = 0;

- instruction_list[issuedInstr].issue = clockCycles+1;

- issueSuccessful = true;

- if (strcmp(instruction_list[issuedInstr].type, "ADDD") == 0) {

- strcpy(add_rs[l].name, "ADDD");

- add_rs[l].cycles_required = addCycles;

- }

- else {

- add_rs[l].cycles_required = subCycles;

- strcpy(add_rs[l].name, "SUBD");

- }

- }

- }

- }

- }

- // if loading

- else if (strcmp(instruction_list[issuedInstr].type, "LD") == 0) {

- for (int l=0; l < loadReservationStations; l++) {

- if (issueSuccessful == false) {

- if (load_rs[l].busy == false) {

- for (int i=0; i < numRegisters; i++) {

- // load value

- load_rs[l].address = instruction_list[issuedInstr].load;

- // destination register

- if (strncmp(instruction_list[issuedInstr].dest_reg, data_registers[i].name, 2) == 0) {

- data_registers[i].tag = load_rs[l].num;

- }

- }

- load_rs[l].busy = true;

- load_rs[l].cycles_required = loadCycles;

- instruction_list[issuedInstr].rs = load_rs[l].num;

- load_rs[l].cycle_count = 0;

- instruction_list[issuedInstr].issue = clockCycles+1;

- issueSuccessful = true;

- }

- }

- }

- }

- // if storing

- else if (strcmp(instruction_list[issuedInstr].type, "SD") == 0) {

- for (int l=0; l < storeReservationStations; l++) {

- if (issueSuccessful == false) {

- if (store_rs[l].busy == false) {

- for (int i=0; i < numRegisters; i++) {

- if (strcmp(instruction_list[issuedInstr].dest_reg, data_registers[i].name) == 0) {

- if (data_registers[i].tag == 0) {

- store_rs[l].address = data_registers[i].data;

- }

- else {

- store_rs[l].tag = data_registers[i].tag;

- }

- }

- // destination register

- if (strncmp(instruction_list[issuedInstr].reg_j, data_registers[i].name, 2) == 0) {

- data_registers[i].tag = store_rs[l].num;

- }

- }

- store_rs[l].busy = true;

- store_rs[l].cycles_required = storeCycles;

- instruction_list[issuedInstr].rs = store_rs[l].num;

- store_rs[l].cycle_count = 0;

- instruction_list[issuedInstr].issue = clockCycles+1;

- issueSuccessful = true;

- }

- }

- }

- }

- // if multiplication or division

- else {

- // adding to reservation station

- for (int l=0; l < mulReservationStations; l++) {

- if (issueSuccessful == false) {

- if (mul_rs[l].busy == false) {

- for (int i=0; i < numRegisters; i++) {

- // destination register

- if (strcmp(instruction_list[issuedInstr].dest_reg, data_registers[i].name) == 0) {

- data_registers[i].tag = mul_rs[l].num;

- }

- // j operand

- if (strcmp(instruction_list[issuedInstr].reg_j, data_registers[i].name) == 0) {

- if (data_registers[i].tag == 0) {

- mul_rs[l].data_j = data_registers[i].data;

- }

- else {

- mul_rs[l].tag_j = data_registers[i].tag;

- }

- }

- // k operand

- if (strncmp(instruction_list[issuedInstr].reg_k, data_registers[i].name, 2) == 0) {

- if (data_registers[i].tag == 0) {

- mul_rs[l].data_k = data_registers[i].data;

- }

- else {

- mul_rs[l].tag_k = data_registers[i].tag;

- }

- }

- }

- mul_rs[l].busy = true;

- instruction_list[issuedInstr].rs = mul_rs[l].num;

- mul_rs[l].cycle_count = 0;

- instruction_list[issuedInstr].issue = clockCycles+1;

- issueSuccessful = true;

- if (strcmp(instruction_list[issuedInstr].type, "MULTD") == 0) {

- strcpy(mul_rs[l].name, "MULTD");

- mul_rs[l].cycles_required = multCycles;

- }

- else {

- mul_rs[l].cycles_required = diviCycles;

- strcpy(mul_rs[l].name, "DIVD");

- }

- }

- }

- }

- }

- }

- // ================== COMPLETING AND EXECUTING INSTRUCTION CHECK ==================

- // ordered by increasing reservation station number

- cdb_busy = false;

- // add reservations

- for (int i=0; i < addReservationStations; i++){

- // executing instructions

- if (add_rs[i].busy and add_rs[i].executing) {

- if (add_rs[i].cycle_count < add_rs[i].cycles_required) {

- add_rs[i].cycle_count += 1;

- }

- }

- // setting mark of beginning of execution

- for (int j=0; j < issuedInstr; j++) {

- if (instruction_list[j].rs == add_rs[i].num) {

- if (add_rs[i].cycle_count==1) {

- instruction_list[j].start = clockCycles+1;

- }

- }

- }

- // completing instruction

- if (add_rs[i].cycle_count == add_rs[i].cycles_required) {

- for (int j=0; j < issuedInstr; j++) {

- // just tracking completion cycle - simply for bookkeeping

- if (instruction_list[j].rs == add_rs[i].num) {

- if (instruction_list[j].completion == -1) {

- instruction_list[j].completion = clockCycles+1;

- completed_rs = add_rs[i].num;

- completedInstr += 1;

- }

- }

- }

- //writing to CDB

- if (!cdb_busy) {

- if (strcmp(add_rs[i].name, "ADDD") == 0) {

- cdb_data = add_rs[i].data_j + add_rs[i].data_k;

- }

- else {

- cdb_data = add_rs[i].data_j - add_rs[i].data_k;

- }

- cdb_busy = true;

- //completed_rs = add_rs[i].num;

- }

- }

- }

- // mult reservations

- for (int i=0; i < mulReservationStations; i++) {

- // executing instructions

- if (mul_rs[i].busy and mul_rs[i].executing) {

- // only increment if not yet reached

- if (mul_rs[i].cycle_count < mul_rs[i].cycles_required) {

- mul_rs[i].cycle_count += 1;

- }

- }

- // setting mark of beginning of execution

- for (int j=0; j < issuedInstr; j++) {

- if (instruction_list[j].rs == mul_rs[i].num) {

- if (mul_rs[i].cycle_count==1) {

- instruction_list[j].start = clockCycles+1;

- }

- }

- }

- // completing instruction

- if (mul_rs[i].cycle_count == mul_rs[i].cycles_required) {

- for (int j=0; j < issuedInstr; j++) {

- // just tracking completion cycle - simply for bookkeeping

- if (instruction_list[j].rs == mul_rs[i].num) {

- if (instruction_list[j].completion == -1) {

- instruction_list[j].completion = clockCycles+1;

- completed_rs = mul_rs[i].num;

- completedInstr += 1;

- }

- }

- }

- //writing to CDB

- if (!cdb_busy) {

- if (strcmp(mul_rs[i].name, "MULTD") == 0) {

- cdb_data = mul_rs[i].data_j * mul_rs[i].data_k;

- }

- else {

- cdb_data = mul_rs[i].data_j / mul_rs[i].data_k;

- }

- cdb_busy = true;

- //completed_rs = mul_rs[i].num;

- }

- }

- }

- // load reservation

- for (int i=0; i < loadReservationStations; i++) {

- // executing instructions

- if (load_rs[i].busy and load_rs[i].executing) {

- // only increment if not yet reached

- if (load_rs[i].cycle_count < load_rs[i].cycles_required) {

- load_rs[i].cycle_count += 1;

- }

- }

- // setting mark of beginning of execution

- for (int j=0; j < issuedInstr; j++) {

- if (instruction_list[j].rs == load_rs[i].num) {

- if (load_rs[i].cycle_count==1) {

- instruction_list[j].start = clockCycles+1;

- }

- }

- }

- // completing instruction

- if (load_rs[i].cycle_count == load_rs[i].cycles_required) {

- for (int j=0; j < issuedInstr; j++) {

- // just tracking completion cycle - simply for bookkeeping

- if (instruction_list[j].rs == load_rs[i].num) {

- if (instruction_list[j].completion == -1) {

- instruction_list[j].completion = clockCycles+1;

- completed_rs = load_rs[i].num;

- completedInstr += 1;

- }

- }

- }

- //writing to CDB

- if (!cdb_busy) {

- cdb_data = load_rs[i].address;

- cdb_busy = true;

- //completed_rs = load_rs[i].num;

- }

- }

- }

- // store reservation

- for (int i=0; i < storeReservationStations; i++) {

- // executing instructions

- if (store_rs[i].busy and store_rs[i].executing) {

- cout << "cycle count" << store_rs[i].cycle_count << endl;

- cout << "cycles required" << store_rs[i].cycles_required << endl;

- // only increment if not yet reached

- if (store_rs[i].cycle_count < store_rs[i].cycles_required) {

- store_rs[i].cycle_count += 1;

- }

- }

- // setting mark of beginning of execution

- for (int j=0; j < issuedInstr; j++) {

- if (instruction_list[j].rs == store_rs[i].num) {

- if (store_rs[i].cycle_count==1) {

- instruction_list[j].start = clockCycles+1;

- }

- }

- }

- //completing instruction

- if (store_rs[i].cycle_count == store_rs[i].cycles_required) {

- for (int j=0; j < issuedInstr; j++) {

- // just tracking completion cycle - simply for bookkeeping

- if (instruction_list[j].rs == store_rs[i].num) {

- if (instruction_list[j].completion == -1) {

- instruction_list[j].completion = clockCycles+1;

- completed_rs = store_rs[i].num;

- completedInstr += 1;

- }

- }

- }

- //writing to CDB

- if (!cdb_busy) {

- cdb_data = store_rs[i].address;

- cdb_busy = true;

- //completed_rs = store_rs[i].num;

- }

- }

- }

- for (int l=0; l < addReservationStations; l++) {

- if (add_rs[l].busy == true) {

- if (add_rs[l].tag_j == 0 and add_rs[l].tag_k == 0) {

- add_rs[l].executing = true;

- }

- }

- }

- for (int l=0; l < loadReservationStations; l++){

- if (load_rs[l].busy == true){

- load_rs[l].executing = true;

- }

- }

- for (int l=0; l < storeReservationStations; l++) {

- if (store_rs[l].busy == true) {

- if (store_rs[l].tag == 0) {

- store_rs[l].executing = true;

- }

- }

- }

- for (int l=0; l < mulReservationStations; l++) {

- if (mul_rs[l].busy == true) {

- if (mul_rs[l].tag_j == 0 and mul_rs[l].tag_k == 0) {

- mul_rs[l].executing = true;

- }

- }

- }

- // ================== PRINTING TO CONSOLE ==================

- printInstructionStatus("test", 6);

- for (int j=0; j < lineCount; j++) {

- printElement(instruction_list[j].type, 15);

- if (strcmp(instruction_list[j].type, "LD") == 0) {

- printElement(instruction_list[j].load, 6);

- printElement(" ", 8);

- }

- else {

- printElement(instruction_list[j].reg_j[0], 0);

- printElement(instruction_list[j].reg_j[1], 6);

- printElement(instruction_list[j].reg_k[0], 0);

- printElement(instruction_list[j].reg_k[1], 6);

- }

- if (instruction_list[j].issue == -1) {

- printElement(" ", 8);

- }

- else printElement(instruction_list[j].issue, 8);

- if (instruction_list[j].start!=0)

- {

- printElement(instruction_list[j].start,8);

- }

- else printElement(" ", 8);

- if (instruction_list[j].completion == -1) {

- printElement(" ", 12);

- }

- else printElement(instruction_list[j].completion, 12);

- if (instruction_list[j].written == -1) {

- printElement(" ", 0);

- }

- else printElement(instruction_list[j].written, 0);

- cout << endl;

- }

- printStationStatus("test", 6);

- for (int i=0; i < addReservationStations; i++) {

- if (add_rs[i].cycles_required == -999) {

- printElement("", 8);

- }

- else {

- printElement(add_rs[i].cycles_required - add_rs[i].cycle_count, 8);

- }

- printElement("[Add", 0);

- printElement(add_rs[i].num, 0);

- printElement("]", 2);

- if (add_rs[i].busy == 0) {

- printElement(" ", 8);

- printElement(" ", 8);

- printElement(" ", 8);

- printElement(" ", 8);

- }

- else {

- printElement(add_rs[i].busy, 8);

- printElement(add_rs[i].name, 8);

- printElement(add_rs[i].tag_j, 8);

- printElement(add_rs[i].data_j, 8);

- printElement(add_rs[i].tag_k, 8);

- printElement(add_rs[i].data_k, 8);

- }

- cout << endl;

- }

- for (int i=0; i < mulReservationStations; i++) {

- if (mul_rs[i].cycles_required == -999) {

- printElement("", 8);

- }

- else {

- printElement(mul_rs[i].cycles_required - mul_rs[i].cycle_count, 8);

- }

- printElement("[Mult", 0);

- printElement(mul_rs[i].num-3, 0);

- printElement("]", 2);

- if (mul_rs[i].busy == 0) {

- printElement(" ", 8);

- printElement(" ", 8);

- printElement(" ", 8);

- printElement(" ", 8);

- }

- else {

- printElement(mul_rs[i].busy, 8);

- printElement(mul_rs[i].name, 8);

- printElement(mul_rs[i].tag_j, 8);

- printElement(mul_rs[i].data_j, 8);

- printElement(mul_rs[i].tag_k, 8);

- printElement(mul_rs[i].data_k, 8);

- }

- cout << endl;

- }

- printLoadStatus("test", 6);

- for (int i=0; i < loadReservationStations; i++) {

- printElement("", 8);

- printElement("[Load", 0);

- printElement(load_rs[i].num-5, 0);

- printElement("]", 2);

- printElement(load_rs[i].busy, 10);

- if (load_rs[i].busy == true) {

- printElement(load_rs[i].address, 0);

- }

- cout << endl;

- }

- printStoreStatus("test", 6);

- for (int i=0; i < storeReservationStations; i++) {

- printElement("", 8);

- printElement("[Stor", 0);

- printElement(store_rs[i].num-8, 0);

- printElement("]", 2);

- printElement(store_rs[i].busy, 10);

- if (store_rs[i].busy == true) {

- printElement(store_rs[i].address, 0);

- }

- cout << endl;

- }

- printRegisterStatus("test", 6);

- printElement(clockCycles+1, 8);

- for (int i=0; i < numRegisters; i++) {

- if (data_registers[i].tag == 0) {

- printElement(data_registers[i].data, 8);

- }

- else {

- printElement("[", 0);

- printElement(data_registers[i].tag, 0);

- printElement("]", 8);

- }

- }

- cout << endl << endl;

- // ================== INCREMENT COUNTERS ==================

- if (issueSuccessful) {

- issuedInstr += 1;

- issueSuccessful = false;

- }

- clockCycles += 1;

- }

- return 0;

- }

- // ================== PRINT FUNCTIONS ==================

- void header(int n)

- {

- cout << "Cycle " << n << endl << endl;

- }

- template<typename T> void printElement(T t, const int& width)

- {

- const char separator    = ' ';

- cout << left << setw(width) << setfill(separator) << t;

- }

- template<typename T> void printInstructionStatus(T t, const int& width)

- {

- cout << "Instruction Status:" << endl;

- printElement("Instruction", 15);

- printElement("j", 6);

- printElement("k", 6);

- printElement("Issue", 8);

- printElement("Start", 8);

- printElement("Completion", 12);

- printElement("Written", 0);

- cout << endl;

- }

- template<typename T> void printLoadStatus(T t, const int& width)

- {

- cout << endl << "Load Status:" << endl;

- printElement("", 8);

- printElement("Name", 8);

- printElement("Busy", 10);

- printElement("Address", 0);

- cout << endl;

- }

- template<typename T> void printStoreStatus(T t, const int& width)

- {

- cout << endl << "Store Status:" << endl;

- printElement("", 8);

- printElement("Name", 8);

- printElement("Busy", 10);

- printElement("Address", 0);

- cout << endl;

- }

- template<typename T> void printStationStatus(T t, const int& width)

- {

- cout << endl << "Reservation Stations:" << endl;

- printElement("Time", 8);

- printElement("Name", 8);

- printElement("Busy", 8);

- printElement("Op", 8);

- printElement("Qj", 8);

- printElement("Vj", 8);

- printElement("Qk", 8);

- printElement("Vk", 8);

- cout << endl;

- }

- template<typename T> void printRegisterStatus(T t, const int& width)

- {

- cout << endl << "Register Result Status:" << endl;

- printElement("Clock", 8);

- printElement("R0", 8);

- printElement("R2", 8);

- printElement("R4", 8);

- printElement("R6", 8);

- printElement("R8", 8);

- printElement("R10", 8);

- cout << endl;

- }